RY-LPITE-802GBTME, industrial Layer-2/3 IP-switch for video with PoE++ 90W and two optical UpLinks

# General description

## Short description

## Industrial PoE Gigabit IP switch, Layer 2/3 with 8 electrical ports 10/100/1000BaseTX with PoE++, two UpLink ports for 100/1000-SFP, manageable

## Special features

- DMS (Device Management System), the switch has an integrated network monitoring and control system, which gives the user a good overview of the entire network in a very simple way.

- PoE 802.3af/at/bt with max. 90W at least at four ports

- Fan-less switch, no noise

- High network security

- Two optical ports for UpLinks

- High backplane performance

- Layer3, static routing

# Special features for video networks

- Active monitoring of the camera
The switch continuously monitors PoE-powered cameras. In the event of a camera failure, the switch automatically restarts the camera. At the same time, the switch sends an SNMP message.

- Active monitoring of the PoE supply
If, for example, too much power is demanded from the switch due to a defective camera, the switch alerts via SNMP.

- Active integration of the switch in video management systems
For the popular video management systems Milestone and Siveillance Video, SW modules are available which allow direct integration of the switch management and the DMS into this VMS

- Jumbo frames even at 100 Mbps
Jumbo Frames up to 9'600Bytes are also supported at 100MBit/s.

# Device Management System DMS

The switch has an integrated network management system with which the following functions are possible:

- Dynamic schematic representation of the network

- The whole network is shown schematically. All switches, servers and end devices are shown individually. The representation can be switched between a schematic view and a tabular view.

- All displays can be exported as snapshot.

- Display of the IP addresses of all network and end devices.

- Display of the device type, switch, IP camera, server, etc.

- Simultaneous upload of new firmware via the DMS of all identical switches in the network.

- Analysis of the data cables connected to the switch

# Safety features

- Certified authentication HTTPS
It must be possible to install a private HTTPS key for management access.

- User management
The rights of the users must be freely adjustable on at least 15, freely adjustable levels.

- MAC address table
It must be possible to manage the MAC table automatically and manually. Static entries must be possible.

- ARP table
The ARP must be able to be managed dynamically and statically. It must be possible to convert a dynamically created table into a static one.

- IP source guard
The machine must be capable of checking the MAC address in combination with the IP address.

- Private VLANs
It must be possible to separate terminal devices within a VLAN with private VLANs.

- ACL access control
It must be possible to define rules and conditions for incoming packets per port. The rules must be able to operate according to either the authorisation or the exclusion procedure. Filtering can be based on source and destination addressing and includes MAC, IP and VLAN ID.

- RADIUS and TACACS+ authentication
Authorisation and billing. MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions. Supports IGMP-RADIUS based 802.1X. Dynamic VLAN assignment

# Technical specification

### Port list

Optical ports	2 SFP-slots for SFP (Mini GBIC) fort he following interfaces:
1000BaseSX (Gigabit Multimode)
1000BaseLX (Gigabit Singlemode)
1000BaseLX bidi (Gigabit Singlemode only one fibre)
100BaseFX Multimde for one or two fibres
100BaseFX Singlemode for one or two fibres

Electrical ports	8 x 10/100/1000BaseTX (RJ45) with PoE 802.3af/at/bt
Total 240W PoE power

Console port	RS232, CLI, RJ45

### Hardware

Architecture	Store and forward

Switch Backplane	min. 24GBit/s

MAC adresses	8K

Jumbo Frame	9600kBytes

### Management

System configuration	Web Browser, DMS, console CLI, SNMPv1, v2c and v3 (USM), http, HTTPS, USB-Port Telnet
It must be possible to lock individual configuration methods.

Port configuration	Port disable/enable. Auto-negotiation 10/100/1000Mbps. Flow Control disable/enable. Data rate control on each port. Max. framesize, Power Control, PoE schedule.

Port status	Display per port Speed Link Status, Flow Control Status. Auto negotiation status, trunk status.

VLAN	802.1Q Tagged Based VLAN ,up to 64 VLAN groups, Q-in-Q, private VLAN

Link Aggregation	IEEE 802.3ad LACP / Static Trunk, supports 18 groups of 8-port trunks or static trunk

QoS	Traffic classification based, Strict priority and WRR, 8-level priority for switching, Port number, 802.1p priority, - DS/TOS field in IP Packet

IGMP Snooping	IGMP (v1/v2) Snooping, up to 256K multicast groups, IGMP Querier mode support, MLD v1/v2, Proxy

Access Control Liste	IP-Based ACL/MAC-Based ACL, 256 entries, VLAN ID, u.v.a.

SNMP MIBs	v1, v2c and v3 with fortraps

### Standards			IEEE 802.3 10Base-T

IEEE 802.3u 100Base-TX/100BASE-FX

IEEE 802.3z Gigabit SX/LX

IEEE 802.3ab Gigabit 1000T

IEEE 802.3x Flow Control and Back pressure

IEEE 802.3ad Port trunk with LACP

IEEE 802.1d Spanning tree protocol

IEEE 802.1w Rapid spanning tree protocol

IEEE 802.1s Multiple spanning tree protocol

IEEE 802.1p Class of service

IEEE 802.1Q VLAN Tagging

IEEE 802.1x Port Authentication Network Control

IEEE 802.1ab LLDP

IEEE 802.3af/at/bt Power over Ethernet

IEEE 802.az Energy Efficient Ethernet

Free fall: IEC60068-2-32

Schock: IEC60068-2-27

Vibration: IEC60068-2-6

Railway: EN0121-4, EN50155

Electrical and thermal values

Power voltage	48-57VDC
Redundant Power input must be possible
When one of the two power inputs fails a closure contact must switch

Power need without PoE	15W

Power need with PoE	495W

Operating temperature:	-45°C to +75°C

Storing temperature:	-45°C to +75°C

Humidity:	20% to 95% relative humidity non-condensing

Manufacturer	barox Kommunikation